| 120VA potted power transformer P/N: | A1461544 |
| --- | --- |
| Power: | 120VA |
| Rated input: | 115V, 115V/50Hz |
| Rated output: | (no load)220V-0-220V, 15.5V, 8.7V |
| Insulation resistance: | ≥500MΩ |
| Insulation strength: | Primary-Secondary: 4000V/2mA/2 S Secondary-pot: 2500V/2mA/2S |
| Insulation ranking: | Class B |
| Temperature rise: | ≤65℃ |
| Lead wire: | ①②③④: 300mm ⑤⑥⑦⑧⑨⑩⑪: 300mm |

⑧

⑨

0V

15.5V

⑥

⑦

0V

220V

⑤

220V

⑩

⑪

0V

8.7V

Schematic

①

②

0V

115V

③

④

0V

115V

Potted power transformer

www.dibao.com

| Dimension (mm) | D | 125 |
| --- | --- | --- |
|  | H | 66 |
| Mounting (mm) | Center screw M8 |  |
| Weight (Kg) |  |  |

Potted power transformer

Transformer enclosed in 2mm thickness steel pot

H

D

Center screw

2mm

www.dibao.com

www.dibao.com