| 41VA potted power transformer P/N: | A1461543 |
| --- | --- |
| Power: | 41VA |
| Rated input: | 115V, 115V/50Hz |
| Rated output: | (no load)16V-0-16V, 9V |
| Insulation resistance: | ≥500MΩ |
| Insulation strength: | Primary-Secondary: 4000V/2mA/2 S Secondary-pot: 2500V/2mA/2S |
| Insulation ranking: | Class B |
| Temperature rise: | ≤65℃ |
| Lead wire: | ①②③④: 300mm ⑤⑥⑦⑧⑨: 300mm |

⑥

⑦

0V

16V

⑤

16V

Schematic

①

②

0V

115V

③

④

0V

115V

⑧

⑨

0V

9V

www.dibao.com

Potted power transformer

| Dimension (mm) | D | 87 |
| --- | --- | --- |
|  | H | 53 |
| Mounting (mm) | Center screw M5 |  |
| Weight (Kg) |  |  |

Potted power transformer

Transformer enclosed in 2mm thickness steel pot

D

Center screw

2mm

H

www.dibao.com

www.dibao.com